GC Retention and Peak-Area Reasoning

Calculate peak metrics and distinguish uncorrected signal from composition.

STUDENT PRACTICE  •  SIMULATED DATA

Name: _____________________________________    Date: _________________

All signals and numbers in this worksheet are simulated for teaching. GC uncorrected area percentages are not validated mole or mass compositions.

1.  Peak A has tR = 2.10 min and area 420 AU. Peak B has tR = 5.40 min and area 580 AU. Calculate the time separation and area% A.

___________________________________________________________________________________

___________________________________________________________________________________

2.  Three peaks have areas 215, 435 and 350 AU. Determine area% of the first peak.

___________________________________________________________________________________

___________________________________________________________________________________

3.  Peak A has area 600 AU and peak B 400 AU. Response-per-mole factors from validated standards are A=2.00 and B=1.00 AU/mol in consistent scaled units. Calculate the corrected mole fraction of A.

___________________________________________________________________________________

___________________________________________________________________________________

4.  Why is a 70% uncorrected GC area fraction not necessarily 70 mol%, 70 mass% or 70% purity?

___________________________________________________________________________________

___________________________________________________________________________________

5.  A sample peak elutes at 4.12 min; a candidate standard elutes at 4.10 min. Is identity proven?

___________________________________________________________________________________

___________________________________________________________________________________

6.  Two peaks have tR values separated by 0.40 min but substantially overlapping widths. Does separation in tR alone establish baseline resolution?

___________________________________________________________________________________

___________________________________________________________________________________